Control Runs: all start 10 Aug 6 UTC

fdda in d01 (including PBL); fdda in d02 (no PBL) for entire time period

all use the same namelist

ECMWF: We use NCAR ECMWF analysis fields (available from NCAR’s Research Data

(RDA), https://rda.ucar.edu/) for meteorological initial and boundary conditions.

(1) EPA 2014

12km EPA 2014 emissions for d02 (EPA2014_test; only for 12 August, repeat those for also 10 and 11 August); no emis for MVK, MACR and CH3COOH

CMAQ_EPA2014 for d01

on modeling1: WRF_EPA2014

09/12: found an error in emissions. Starting 13 Aug 0 UTC there are some other high resolution emissions being used.

(2) S05

CMAQ_S05 for d02

CMAQ_EPA2014 for d01

on modeling1: WRF_S05

(3) NEI 2011 adj

NEI 2011 for d01 and d02 (TOTAL_MobRed_adj_noDIURNAL_doubleOGNOx)

d01: mobile -50%

d02: mobile-50%, OG * 2 (but not C2H6 and C3H8)

no diurnal cycle in OG emissions

on modeling1: WRF_NEI2011_adj

Sensitivity runs: based on EPA2014

(1) turn off all HCHO emissions

start on 11 August 6 UTC

(2) turn off all CH3CHO emissions

start on 11 August 6 UTC

(3) Change Physics

WRFV4_S05_d02_MOZAIC_final_adj_EPA2014_phys

lowest layer depth: 12m, 14.m, 16.m

(4) turn off d02 fdda to create trajectories

(5) newWRF – using latest V4.0 and also using te new EPA tool to create emissions – there are some differences to the other EPA2014.

still issing:

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_APIN

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_PM25I

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_PM25J

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_NAI

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_NAJ

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_CLI

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_CLJ

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_CO_A

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_ORGI_A

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_ORGJ_A

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_CO_BB

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_ORGI_BB

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_ORGJ_BB

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_GLY

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_MACR

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_MGLY

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_MVK

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_HCOOH

d01 2014-08-12_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_HONO

(6) newWRF_2xOG
as (5) but double all OG emissions and 4*bigalk, toluene and benzene (but based on EPA EI there are also other non-OG sources for these and even C2H6: total C2H6 with doubling OG increases C2H6 emissions by 1.7; Benzene Times 4 leads to increase of 1.2; Toluene to 1.05; BIGALK by 1.8  I did a simple doubling (/home/pfister/FRAPPE_WRFchemaddOGtoEPA2014.pro

(all the above emission species also still missing)

(7) newWRF_T1

as newWRF but using T1 and SO2 rate correction. also had to change nproc_x to 13 (from 15) to make code run. started on 11 August

d01 2014-08-11_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_HCOOH

d01 2014-08-11_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_CH3CN

d01 2014-08-11_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_CH3COOH

d01 2014-08-11_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_HCN

d01 2014-08-11_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_MVK

d01 2014-08-11_06:00:00  NetCDF error in wrf_io.F90, line        2883  Varname E_MGLY

(8) WRFV4_S05_d02_MOZAIC_final_NEI2011adj_newWRF

as newWRF but using NEI2011 emissions. started on Aug 10 6 UTC

(9) WRFV4_S05_d02_MOZAIC_final_S05_newWRF

as newWRF but using S05 emissions. started on Aug 10 6 UTC

(9) WRFV4_S05_d02_MOZAIC_final_EPA2014_newWRF_phys

as newWRF but using changing physics started on Aug 10 6 UTC

created new wrfinput and wrfbdy and saved setup on Globus

(10) newWRF_noHCHO: no direct emissions of HCHO in d02

(11) newWRF_noCH3CHO: no direct emissions of CH3CHO in d02

12/12/2018: start a new suite of runs

Discovered issue in wrfinput levels 38-top for all runs except phys run. Not using UBC yet outputs look fairly reasonable for EPA2014_newRun. Definitely problem for NEI2011 and S05 and 2xOG. Not sure what and if issue. T1 should also have been fine.

WRFreal_3.9_EPA2014_mods has corrected wrfinputs

switch all to RAQMS input files. for wrfinput_d01, but  keep wrfinput_d02 for d02

created all wrfinputs in separate _raqms and _ directories

all runs in bold are finished.

other changes: only output d01 every 6 hours. No writing of wrfrst.

and change all to: 
guv                                            = 0.0003, 0.0003, 0.0003,

gt                                             = 0.0003, 0.0003, 0.0003,

plus apply fdda to both domains

WRFreal_3.9_EPA2014_phys_2: ERA-interim met_em files, changes in physics, create wrfinputs with V4.0 -> will not need to set use_old_input

https://rda.ucar.edu/datasets/ds626.0 and https://rda.ucar.edu/datasets/ds627.0

ECMWF: https://rda.ucar.edu/datasets/ds113.0/

For phys runs ensure to use correct fdda files!

(1) newWRF_BC: uses MOZART IC and BC, but  increase ozone in wrfinput_d01 and wrfinput_d02 for 10 Aug 6 UTC by 15 ppb at all altitudes., with fix

ozone increases but at all altitudes and that means that surface ozone will be off (not yet tested). – if stick with the current setups then could use to demonstrate the impact of BC (or use RAQMS/MOZART for this).

(2) WRFV4_S05_d02_MOZAIC_final_adj_EPA2014_newWRF_fix: with fixed MOZART IC and BC.   identical to previous run.

(3) WRFV4_S05_d02_MOZAIC_final_adj_EPA2014_newWRF_RAQMS: as WRFV4_S05_d02_MOZAIC_final_adj_EPA2014_newWRF but using RAQMS for IC and BC.

has higher ozone, but vertical distribution

(4) WRFV4_S05_d02_MOZAIC_final_NEI2011adj_newWRF

as newWRF but using NEI2011 emissions. started on Aug 10 6 UTC

(5) WRFV4_S05_d02_MOZAIC_final_S05_newWRF

as newWRF but using S05 emissions. started on Aug 10 6 UTC

6) newWRF_2xOG
as (5) but double all OG emissions and 4*bigalk, toluene and benzene (but based on EPA EI there are also other non-OG sources for these and even C2H6: total C2H6 with doubling OG increases C2H6 emissions by 1.7; Benzene Times 4 leads to increase of 1.2; Toluene to 1.05; BIGALK by 1.8  I did a simple doubling (/home/pfister/FRAPPE_WRFchemaddOGtoEPA2014.pro

(all the above emission species also still missing)

These two are with settings from WRFV4_S05_d02_MOZAIC_final_adj_EPA2014_newWRF and need to compared to that one:

(10) newWRF_noHCHO: no direct emissions of HCHO in d02, with fix

(11) newWRF_noCH3CHO: no direct emissions of CH3CHO in d02, with fix

These two are with settings from WRFV4_S05_d02_MOZAIC_final_adj_EPA2014_newWRF_RAQMS and need to compared to that one:  Emissions only turned off in d02

(10) newWRF_RAQMS_noHCHO: no direct emissions of HCHO in d02, with fix

(11) newWRF_RAQMS_noCH3CHO: no direct emissions of CH3CHO in d02, with fix   started

Base runs: 
&domains

time_step                           = 36,

e_vert                              = 51,51,    30,

p_top_requested                     = 5000,

num_metgrid_levels                  = 26,

/

&physics

mp_physics                          = 8,     8,     3,

ra_lw_physics                       = 4,     4,     1,

ra_sw_physics                       = 4,     4,     1,

sf_sfclay_physics                   = 1,1,     1,

sf_surface_physics                  =  2,2,     2,

bl_pbl_physics                      = 1, 1,  1,

bldt                                = 0,     0,     0,

cu_physics                          = 3, 3,     0,

cudt                                = 0,     0,     5,

cu_diag                             = 1, 1,

cugd_avedx = 1

surface_input_source                = 1,

num_soil_layers                     = 2,

num_land_cat                        = 21,

sf_urban_physics                    = 1, 1,     0,

/

&fdda

grid_fdda                                      = 1, 1, 0,

gfdda_end_h                                    = 99999, 6, 120,

gfdda_interval_m                               = 360, 360, 360,

if_no_pbl_nudging_uv                           = 0, 1, 0,

if_no_pbl_nudging_t                            = 0, 1, 1,

if_no_pbl_nudging_q                            = 0, 1, 1,

if_zfac_uv                                     = 0, 1, 1,

k_zfac_uv                                      = 10, 50, 10,

if_zfac_t                                      = 0, 0, 0,

k_zfac_t                                       = 10, 10, 10,

if_zfac_q                                      = 0, 0, 0,

k_zfac_q                                       = 10, 10, 10,

guv                                            = 0.0003, 0.0003, 0.0003,

gt                                             = 0.0003, 0.0003, 0.0003,

gq                                             = 0.0001, 0.0001, 0,

if_ramping                                     = 1,

dtramp_min                                     = 60.0,

io_form_gfdda                                  = 2,

Difference base and Phys:

>  e_vert                              = 37,37,    30,

86a86,93

>  eta_levels                          = 1.0000, 0.9985, 0.9970, 0.9950, 0.9930,

>                                         0.9910, 0.9880, 0.9850, 0.9800, 0.9700,

>                                        0.9600, 0.9500, 0.9400, 0.9300, 0.9100,

>                                        0.8900, 0.8700, 0.8400, 0.8000, 0.7600,

>                                        0.7200, 0.6800, 0.6400, 0.6000, 0.5500,

>                                         0.5000, 0.4500, 0.4000, 0.3500, 0.3000,

>                                         0.2500, 0.2000, 0.1500, 0.1000, 0.0600,

>                                         0.0270, 0.0000

94c101

---

>  sf_sfclay_physics                   = 2,2,     1,

---

>  bl_pbl_physics                      = 5,5,  1,

>  cu_physics                          = 5,5,     0,

---

>  sf_urban_physics                    = 0,0,     0,

---

>  gfdda_end_h                                    = 99999,99999, 120,

Difference base and Phys_2:

>  e_vert                              = 47,47,    30,

86d84

>  mp_physics                          = 10,10,     3,

>  radt                                = 12,    4,    30,

>  sf_sfclay_physics                   =1,1,     1,

>  sf_surface_physics                  = 2,2,     2,

>  bl_pbl_physics                      = 5,5,  1,

>  cu_physics                          = 5,5,     0,

---

>  cugd_avedx = 3

>  !isfflx                              = 1,

>  !ifsnow                              = 1,

>  !icloud                              = 1,

>  num_soil_layers                     = 4,

---

>  sf_urban_physics                    = 1,1 ,     0,

>  gfdda_end_h                                    = 99999, 99999, 120,

>  chem_adv_opt                       = 1,1,      1,

>  moist_adv_opt                       = 1,1 ,      1,